# Linux Kernel Rebuild Image for Driver Modification

# STMMAC Driver Modification via Full Image Rebuild (For IQ9075)

## Objective

Modify the STMMAC driver and rebuild the full kernel image instead of inserting module manually to the IQ9 board.

## Step 1: Navigate to Working Directory

cd ~/qclinux

## Step 2: Source Build Environment

source setup-environment

## Step 3: Modify Driver Source Code (stmmac)

cd tmp-glibc/work-shared/qcs9075-iq-9075-evk/kernel-source/drivers/net/ethernet/stmicro/stmmac/

vi stmmac_main.c

Add printk or any debugging statements.

## Step 4: Rebuild Kernel/Image

cd ~/qclinux/build-qcom-wayland

bitbake qcom-multimedia-image -C compile

-C compile forces BitBake to rerun the do_compile task

## Step 5: Verify Build Output

ls -lh ~/qclinux/build-qcom-wayland/tmp-glibc/deploy/images/qcs9075-iq-9075-evk/qcom-multimedia-image/Image

ls -lh ~/qclinux/build-qcom-wayland/tmp-glibc/deploy/images/qcs9075-iq-9075-evk/qcom-multimedia-image/system.img

## Step 6: Flash Image to Board

Using steps to flash the image provided in qualcomm build guide documentation:

- Put the iq9 board into QDL mode and verify using command – lsusb.

- Check if PCAT recognizes the device: pcat –devices

- Go to the image folder, for example:

cd <workspace_path>/build-qcom-wayland/tmp-glibc/deploy/images/qcs6490-rb3gen2-vision-kit/qcom-multimedia-image

- For UFS storage, run the command:

cp ./partition_ufs/gpt_main*.bin ./partition_ufs/gpt_backup*.bin ./partition_ufs/rawprogram[0-9].xml ./partition_ufs/patch*.xml ./partition_ufs/zeros_*sectors.bin ./

- Flash the image using command:

PCAT –PLUGIN SD -DEVICE <device_serial_number> -BUILD "<build_images_path>"" -MEMORYTYPE UFS -FLAVOR asic

For example:

PCAT –PLUGIN SD -DEVICE be116704 -BUILD "<workspace_path>/build-qcom-wayland/tmp-glibc/deploy/images/qcs6490-rb3gen2-vision-kit/qcom-multimedia-image" -MEMORYTYPE UFS -FLAVOR asic

## Step 7: Verify on Target

dmesg | grep stmmac

dmesg | tail -50

## Summary

Driver changes were applied by rebuilding the full kernel image and flashing it to the board instead of loading module separately.