# Kernel Module Modification Exercise on IQ9 board

## Objective

Modify stmmac Ethernet driver, compile it, deploy to target board, and verify using kernel logs.

## Step 1: Setup Build Environment

cd ~/qclinux/
source setup-environment

This sets MACHINE=qcs9075-iq-9075-evk and prepares Yocto build.

## Step 2: Open Kernel Dev Shell

bitbake -c devshell virtual/kernel

This opens a new terminal at kernel source path like:

~/qclinux/build-qcom-wayland/tmp-glibc/work-shared/qcs9075-iq-9075-evk/kernel-source#

## Step 3: Force the deploy task(Don’t build all task)

bitbake linux-qcom-custom -c deploy -f

Forcefully rerun the deploy task for the custom kernel and copy the kernel outputs again to the deploy directory.

## Step 4: Modify Driver Source Code (stmmac)

Go to driver directory:

cd ~/qclinux/build-qcom-wayland/tmp-glibc/work/qcs9075_iq_9075_evk-qcom-linux/linux-qcom-custom/6.6/kernel/drivers/net/ethernet/stmicro/stmmac

Edit file stmmac_main.c by adding print statement as follow:

printk("-----> %s\n", __func__);

Functions modified: stmmac_dvr_probe, __stmmac_open, stmmac_release

## Step 5: Compile Driver Module

make M=$(pwd)/../../../work/qcs9075_iq_9075_evk-qcom-linux/linux-qcom-custom/6.6/kernel/drivers/net/ethernet/stmicro modules

## Step 6: Locate .ko File

To verify if the new .ko got generated after compilation, go to:

cd ~/qclinux/build-qcom-wayland/tmp-glibc/work/qcs9075_iq_9075_evk-qcom-linux/linux-qcom-custom/6.6/kernel/drivers/net/ethernet/stmicro/stmmac

ls -lh stmmac.ko

## Step 7: Transfer Module to Board

scp stmmac.ko root@<board-ip>:/root/

Verify on board:

cd /tmp
ls -lh stmmac.ko

## Step 8: Check Existing Modules

lsmod | grep stmmac

## Step 9: Remove Existing Module and Dependencies

Putting down the ethernet interface to avoid crash: ifconfig eth0 down

Removing dependencies and modules:

rmmod dwmac_qcom_ethqos
rmmod stmmac_platform
rmmod stmmac

## Step 10: Insert New Module

insmod stmmac.ko

## Step 11: Load Dependencies

modprobe stmmac_platform
modprobe dwmac_qcom_ethqos

## Step 12: Bring Interface Up

ifconfig eth0 up

## Step 13: Verify Logs

Dmesg | grep stmmac

dmesg | tail –50

## Observations

Captured dmesg log:

root@qcs9075-iq-9075-evk:~# dmesg | grep stmmac
 [    5.326379] qcom-ethqos 23040000.ethernet eth0: PHY [stmmac-0:1c] driver [Qualcomm QCA8081] (irq=POLL)
 [ 2181.907709] qcom-ethqos 23040000.ethernet eth0: stmmac_dvr_remove: removing driver
 [ 2181.908077] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.908316]  stmmac_bus_clks_config+0x54/0x164 [stmmac]
 [ 2181.908336]  stmmac_runtime_resume+0x1c/0x28 [stmmac_platform]
 [ 2181.908365]  stmmac_dvr_remove+0x48/0x158 [stmmac]
 [ 2181.908378]  devm_stmmac_pltfr_remove+0x24/0x48 [stmmac_platform]
 [ 2181.940533] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.940792] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.941053] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.941175]  devm_stmmac_remove_config_dt+0x20/0x4c [stmmac_platform]
 [ 2181.941239] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.941360]  devm_stmmac_remove_config_dt+0x28/0x4c [stmmac_platform]
 [ 2181.941415] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.941526]  devm_stmmac_remove_config_dt+0x34/0x4c [stmmac_platform]
 [ 2181.941579] Modules linked in: michael_mic overlay tz_log qcom_smcinvoke mem_object si_core_module rfcomm 8021q garp mrp stp llc bnep usb_f_uvc videobuf2_vmalloc uvc videobuf2_dma_sg videobuf2_memops c
 [ 2181.941690]  devm_stmmac_remove_config_dt+0x3c/0x4c [stmmac_platform]
 [ 2197.508342] -----> stmmac_dvr_probe
 [ 2197.630223] -----> __stmmac_open
 [ 2197.704577] qcom-ethqos 23040000.ethernet eth0: PHY [stmmac-0:1c] driver [Qualcomm QCA8081] (irq=POLL)
 [ 2305.156630] -----> stmmac_release

stmmac_dvr_probe → module load
__stmmac_open → interface up
stmmac_release → interface down

## Summary

Successfully modified kernel driver, compiled, deployed, and verified using printk logs.