Input: (1500,)  uint8 bytes

↓

Embedding(256)

↓

Conv1D(64, kernel=5)

↓

ReLU

↓

Conv1D(128, kernel=5)

↓

ReLU

↓

Conv1D(256, kernel=3)

↓

ReLU

↓

GlobalMaxPooling1D

↓

Dense(128)

↓

ReLU

↓

Dense(num_classes)

↓

Softmax

initially we start with understanding the data format we already have generated (.feather.npy) files.

imdad@imdad-Latitude-3440:~/nnDPI$ python

Python 3.10.12 (main, Jan 26 2026, 14:55:28) [GCC 11.4.0] on linux

Type "help", "copyright", "credits" or "license" for more information.

>>> import numpy as np

>>> arr = np.load("ProcessedNumpy/AIMchat1.pcapng.feather.npy")

>>> print(arr.shape)

(178, 1500)

>>> print(arr.dtype)

float32

meaning: we have178 packets, each having 1500 bytes (padded or truncated based on requirement)

The tensor might look loke this:

(samples, features)

(178, 1500)

Example normalization:

0 → 0.0

255 → 1.0

now the array becomes,

arr = [

[0.23, 0.01, 0.87, ..., 0.00],  ← packet 1

[0.45, 0.67, 0.00, ..., 0.12],  ← packet 2

[0.11, 0.98, 0.76, ..., 0.00],  ← packet 3

...

]

2. Now we create labels from the file names (training data):

prepare_dataset.py (check this file)

here, 
X = [ array(178,1500), array(210,1500), array(150,1500), ... ]

y = [ array(178,), array(210,), array(150,), ... ]

3. Now we train the model.

train_cnn_dpi.py (check this file)

why reshaping is required:

Without reshape:

[0.12  0.55  0.23  0.90 ...]

Shape: (1500)

After reshape:

[

[0.12]

[0.55]

[0.23]

[0.90]

...

]

Shape: (1500,1)

Now each byte is treated as a feature channel.

The reshape is needed because the CNN expects a sequence with channels, and the packet is treated as a 1500-step sequence with 1 feature per step.

Now, breaking into training and testing data set:

Training Set: 0.8 * total size

X_train.shape = (27060, 1500, 1)

y_train.shape = (27060,)

Test Set: 0.2 * total size

X_test.shape = (6766, 1500, 1)

y_test.shape = (6766,)

4. Convert the model to TFLite and test on the board:

convert_tflite.py (check this file).

and test_tflite_model.py

example output:

File: AIMchat1.pcapng.feather.npy — 178 packets

Packet 1: class 0 (95.2%)

Packet 2: class 0 (93.8%)

Packet 3: class 0 (96.4%)

Till now, we achieved:

pcap

↓

numpy packet features

↓

CNN training (93% accuracy)

↓

TFLite model

↓

IQ-9075 board inference

5. Convert model to INT8 TFLite:

Now we’ll convert the model to INT8 TFLite, which is the format required to run efficiently on the Hexagon HTP NPU on the IQ-9075.

The process has three steps:

1. Create a representative dataset

2. Convert the model to INT8 TFLite

3. Test the INT8 model on the board

Quantization converts:

float32 weights → int8

float32 activations → int8

On the Training machine:

convert_int8_tflite.py (check this file)

now test it on the board:

test_tflite_int8.py (check this file)

Now,

we do have the Hexagon DSP runtime, but NOT the TFLite Hexagon delegate installed.

If the delegate is not present

Some Qualcomm builds require using the QNN SDK instead of TFLite delegates:

TFLite → ONNX → QNN converter → HTP runtime

To use the NPU:

CNN model
   ↓
ONNX / TF
   ↓
SNPE converter
   ↓
DLC model

↓
snpe-net-run
   ↓
HTP NPU

SNPE uses a DLC model format, not .tflite

Since SNPE converts easiest from ONNX we will first convert the tflite model to ONNX and then to DLC format.

6. Conversion to ONNX and then to DLC:

On the host:

1. Install exporter:

pip install tf2onnx

2. Convert saved model:

python -m tf2onnx.convert --saved-model cnn_dpi_model --output cnn_dpi_model.onnx  --opset 13

we get, cnn_dpi_model.onnx

On the host:

1. Convert ONNX to SNPE DLC:

snpe-onnx-to-dlc --input_network ~/nnDPI/cnn_dpi_model.onnx --output_path ~/nnDPI/cnn_dpi_model.dlc -d keras_tensor 1,1500,1

Here, if you encounter the error, snpe-onnx-to-dlc: command not found,

It means the board does not have the SNPE conversion tools installed.

So, let us download and install the package on the host (not on the board),

https://www.qualcomm.com/developer/software/qualcomm-ai-engine-direct-sdk

here u can get the software and download the .zip file, with name v2.44.0.260225.zip
after unzipping, u will find something like

qairt/

├── 2.32.0.250228/

│   ├── bin

│   ├── lib

│   ├── examples

│   ├── include

now, follow these steps:

sudo apt install libc++1 libc++abi1

sudo apt install clang-14

Then verify, ls /usr/lib/x86_64-linux-gnu/libc++.so.1

check, clang++ --version

next,

cd qairt/2.32.0.250228/bin

source ./envsetup.sh

now create a virtual environment,

python3 -m venv venv --without-pip

source venv/bin/activate

then install pip and required packages:

python3 -m ensurepip --upgrade

python ${QAIRT_SDK_ROOT}/bin/check-python-dependency

install ONNX support:

pip install onnx==1.19.1

pip install onnxruntime==1.23.2

Now check:

which snpe-onnx-to-dlc

example output: /home/imdad/v2.44.0.260225/qairt/2.44.0.260225/bin/x86_64-linux-clang/snpe-onnx-to-dlc

You can also check these steps in the qualcomm docs:

https://docs.qualcomm.com/nav/home/setup_linux.html?product=1601111740009302

Now, coming back to the conversion part,

snpe-onnx-to-dlc --input_network ~/nnDPI/cnn_dpi_model.onnx --output_path ~/nnDPI/cnn_dpi_model.dlc -d keras_tensor 1,1500,1

this generates the .dlc model.

Now, copy the model to the board.

7. Run the DLC model on the board:

1. We create a small raw input tensor file as SNPE expects raw tensor files:

run the script:

import numpy as np

data = np.load("nnDPI/ProcessedNumpy/AIMchat1.pcapng.feather.npy")

packet = data[0].reshape(1,1500,1).astype(np.float32)

packet.tofile("input.raw")

create input list:

input_list.txt

echo  " keras_tensor: input.raw" > input_list.txt

snpe-net-run --container cnn_dpi_model.dlc --input_list input_list.txt --use_dsp

if the run is successful, it creates output/Result_0/output_0.raw

2. Convert the raw output back to prediction:

import numpy as np

output = np.fromfile("output/Result_0/output_0.raw", dtype=np.float32)

print(output)

print("Predicted class:", np.argmax(output))

8. Inorder to fully utilize the NPU, we need to quantize the float32 dlc to int8 dlc model

1. Prepare a calibration dataset:

we need to generate several raw files, run the script

import numpy as np

import os

data = np.load("nnDPI/ProcessedNumpy/AIMchat1.pcapng.feather.npy")

os.makedirs("calib", exist_ok=True)

for i in range(50):

packet = data[i].reshape(1,1,1500).astype(np.float32)

packet.tofile(f"calib/input_{i}.raw")

This creates:

calib/input_0.raw

calib/input_1.raw

...

2. Create a calibration input list

run the script

for f in calib/*.raw; do

echo "keras_tensor:=$f" >> calib_list.txt

done

3. Run SNPE Quantization and verify

run this script where SNPE sdk is installed (likely x86 system)

snpe-dlc-quantize --input_dlc cnn_dpi_model.dlc --input_list calib_list.txt --output_dlc cnn_dpi_model_quantized.dlc

Verify the quantization:

snpe-dlc-info --input_dlc cnn_dpi_model_quantized.dlc

4. Copy the model to the board and run on the NPU with --debug flag to see logs:

snpe-net-run --container cnn_dpi_model_quantized.dlc --input_list input_list.txt --use_dsp --debug