# AI Model Deployment on Qualcomm IQ-9075 (QCS9075)

## 1. Objective

To export an AI model from Qualcomm AI Hub and deploy it on the Dragonwing IQ-9075 EVK (QCS9075 platform), execute inference using the QNN HTP (NPU) backend, and validate performance using on-device testing.

## 2. Target Hardware

Board: Dragonwing IQ-9075 EVK

SoC: Qualcomm QCS9075

Architecture: ARM64 (aarch64)

Operating System: QC_LINUX 1.7

QNN Runtime Version: 2.41

Available Backends: HTP (NPU), CPU, GPU, DSP

## 3. Model Selection

Model: EfficientNet-B0 (Float)

Export Format: QNN Context Binary

## 4. Exporting Model from Development PC

Installed required packages:

pip3 install qai-hub qai-hub-models

Configured authentication using API token:

qai-hub configure --api_token API_TOKEN

Export command used:

python -m qai_hub_models.models.efficientnet_b0.export --target-runtime qnn_context_binary --chipset qcs9075 --compile-options="--qairt_version=2.41" --profile-options="--qairt_version=2.41"

## 5. Transfer to Board

scp efficientnet_b0_float.bin root@BOARD_IP:

## 6. Image Preprocessing

Downloaded sample test image and created input tensor using Python script.

Preprocessing steps: Resize to 224x224, normalize, transpose, add batch dimension.

Generated raw tensor file: input.raw

Python Script:

from PIL import Image

import numpy as np

image = Image.open("test.jpg").resize((224, 224))

image = np.array(image).astype(np.float32) / 255.0

image = np.transpose(image, (2, 0, 1))

image = np.expand_dims(image, axis=0)

print("Shape:", image.shape)

print("Saving input.raw...")

image.tofile("input.raw")

print("Done.")

## 7. Running Inference on IQ-9075

Command used to execute on HTP (NPU):

qnn-net-run --retrieve_context efficientnet_b0_float.bin --backend libQnnHtp.so --input_list input_list.txt --output_dir output

Successful execution confirmed with HtpProvider initialization and graph execution logs.

## 8. Performance Validation

Measured execution time using Linux 'time' command.

HTP backend showed minimal CPU usage, confirming NPU offload.

CPU backend execution was significantly slower, validating hardware acceleration.

## 9. Output Validation

Output logits were generated in raw format and decoded to obtain top-5 classification results.

Python Script For Decoding Output:

import numpy as np

# Load output

logits = np.fromfile("output_0.raw", dtype=np.float32)

logits = logits.reshape(1, 1000)

# Softmax

exp = np.exp(logits)

probs = exp / np.sum(exp)

# Load ImageNet labels

import urllib.request

url = "https://qaihub-public-assets.s3.us-west-2.amazonaws.com/apidoc/imagenet_classes.txt"

labels = urllib.request.urlopen(url).read().decode().splitlines()

# Top 5

top5 = np.argsort(probs[0])[-5:][::-1]

print("\nTop-5 Predictions:\n")

for idx in top5:

print(f"{labels[idx]} : {probs[0][idx]*100:.2f}%")