Networking + AI Use Case

DPI + AI on Qualcomm IQ9 (QCS9075)

nDPI + QNN Runtime + Hexagon NPU — Industrial Anomaly Detection

Embedded Linux Networking  |  AI Inference  |  Roopak  |  2026

# Overview

This document describes a complete Networking + AI pipeline running on the Qualcomm IQ9 (QCS9075) platform. The architecture combines Deep Packet Inspection (DPI) with AI model inference to perform intelligent industrial anomaly detection — classifying network traffic as normal or attack with high accuracy.

## Typical Architecture

Kernel (packet capture)

↓

User-space DPI (libpcap / AF_PACKET)

↓

nDPI Classification

↓

Feature Extraction

↓

AI Model Inference (QNN runtime on NPU)

↓

Decision Engine (Alert / Drop / Log)

# Pipeline Stage Overview

| Stage | Component | What it Does | Where it Runs |
| --- | --- | --- | --- |
| 1 | Kernel RX | Captures packets via IRQ, NAPI, netif_receive_skb | Linux Kernel |
| 2 | AF_PACKET / libpcap | Bridges packet from kernel space to user space | Kernel → User space |
| 3 | nDPI | Protocol classification — HTTP, TLS, Modbus, IEC 61850 | User space library |
| 4 | Feature Extraction | Converts packet data to numerical tensor | User space app |
| 5 | QNN Runtime | AI model inference on HTP/NPU for anomaly detection | Hexagon NPU / CPU / GPU |
| 6 | Decision Engine | Alert, drop, log based on AI model output | User space / iptables / XDP |

| Step 1 | Kernel — Packet Capture |
| --- | --- |

When a packet arrives at the NIC, the Linux kernel processes it through the standard receive path before making it available to user space.

Ethernet PHY → MAC → Driver → Linux Network Stack

Inside kernel RX path:

IRQ fires (NIC signals CPU)

↓

NAPI polls (batch packet processing)

↓

netif_receive_skb() (packet enters kernel stack)

↓

Packet is now inside kernel space

Two choices at this point:

Option A: Let packet go normally to TCP/IP stack

Option B: Copy / redirect to user space for DPI inspection

| Path | Description | Use Case |
| --- | --- | --- |
| Normal TCP/IP | Packet goes to socket buffer, application reads via recv() | Standard networking |
| AF_PACKET tap | Kernel copies packet to raw socket — DPI app inspects it | Monitoring, DPI |
| XDP redirect | Packet intercepted at driver level — fastest path | High-speed DPI, filtering |

| Step 2 | libpcap / AF_PACKET — Bridge to User Space |
| --- | --- |

This is the bridge between kernel space and user space. The DPI application needs raw packet bytes to inspect. Two common methods achieve this.

Method A — AF_PACKET (Raw Socket)

/* Open raw socket — receives ALL Ethernet frames */

int sock = socket(AF_PACKET, SOCK_RAW, htons(ETH_P_ALL));

/* This lets your program receive raw Ethernet frames directly */

/* Taps packets BEFORE full TCP/IP processing */

Method B — libpcap (Wrapper Library)

/* libpcap is built on top of AF_PACKET */

pcap_t *handle = pcap_open_live("eth0", 65535, 1, 1000, errbuf);

/* Simpler API, same result:

Kernel copies packet → User space buffer

DPI application sees raw packet bytes */

| Method | API | Pros | Cons |
| --- | --- | --- | --- |
| AF_PACKET | socket(AF_PACKET,...) | Low level, full control, no extra library | More complex to use |
| libpcap | pcap_open_live(...) | Simple API, widely supported, BPF filters | Slight overhead over AF_PACKET |
| AF_XDP | xsk_socket__create(...) | Zero copy, highest performance | More complex setup |

| Step 3 | nDPI Classification |
| --- | --- |

Now the packet is in user space. Pass it to nDPI for deep protocol classification. nDPI inspects all layers from Ethernet up to Layer 7 application payload.

What nDPI Does Internally

nDPI processing pipeline:

Parse Ethernet header (L2)

↓

Parse IP header (L3)

↓

Parse TCP/UDP header (L4)

↓

Inspect payload (L7 — application layer)

↓

Build flow state (not just single packet)

↓

Output: Flow = HTTPS / Modbus / IEC 61850 / Unknown

Protocols nDPI Can Detect

| Category | Protocols Detected |
| --- | --- |
| Web / IT | HTTP, HTTPS, TLS, DNS, FTP, SMTP, SSH |
| Industrial | Modbus, IEC 61850, GOOSE, Sampled Values, DNP3, EtherNet/IP |
| Streaming | RTP, RTSP, Video streaming protocols |
| Security | Known malware signatures, C2 communication patterns |
| Unknown | Unclassified flows for further AI analysis |

Key Feature — Flow State nDPI builds flow state across multiple packets — not just single packet inspection. After several packets it provides confident classification: Flow = HTTPS, Flow = Modbus. This flow context is critical input for the AI feature extraction stage.

| Step 4 | Feature Extraction — Raw Packets to AI Tensor |
| --- | --- |

Instead of passing raw packet bytes to the AI model, structured numerical features are extracted. This converts network traffic into a numeric vector (tensor) that the AI model can process.

Example Feature Set

| Feature | Example Value | Meaning |
| --- | --- | --- |
| Protocol ID | 91 | HTTPS (nDPI protocol number) |
| Packet Length | 1514 | Size of packet in bytes |
| Flow Duration | 23 ms | How long the flow has been active |
| Packet Count | 10 | Number of packets in this flow |
| Avg Inter-Arrival Time | 2 ms | Time between consecutive packets |
| TLS SNI | google.com | Server Name Indication from TLS handshake |
| SYN/ACK Ratio | 0.95 | Ratio of SYN to ACK packets (attack indicator) |
| Payload Entropy | 7.8 | Randomness of payload — high = encrypted/random |

Converting to Tensor

Network traffic features → Numeric tensor

Example tensor for one flow:

[91, 1514, 23, 10, 2.0, 0.95, 7.8]

│    │    │   │    │    │     │

│    │    │   │    │    │     └── payload entropy

│    │    │   │    │    └──────── SYN/ACK ratio

│    │    │   │    └───────────── inter-arrival time (ms)

│    │    │   └────────────────── packet count

│    │    └────────────────────── flow duration (ms)

│    └─────────────────────────── packet length (bytes)

└──────────────────────────────── protocol ID (HTTPS=91)

| Step 5 | AI Model — QNN Runtime on IQ9 |
| --- | --- |

The feature tensor is fed into an AI model running via the Qualcomm Neural Network (QNN) runtime on the IQ9 platform. The Hexagon NPU (HTP) provides hardware-accelerated inference.

QNN Runtime Backends on QCS9075

| Backend | Hardware | Performance | Best For |
| --- | --- | --- | --- |
| CPU | ARM Cortex-A55 | Baseline | Development, debugging |
| GPU | Adreno GPU | Medium | Parallel inference |
| HTP (Hexagon NPU) | Hexagon DSP | Highest — lowest power | Production deployment |

Types of AI Models Supported

- Anomaly detection — identifies unusual traffic patterns

- Malware classifier — classifies known attack types

- Industrial attack detector — specific to ICS/SCADA threats

- Flow risk scorer — assigns risk score to each flow

Example Model Output

Input tensor:  [91, 1514, 23, 10, 2.0, 0.95, 7.8]

↓

QNN Model Inference (Hexagon NPU)

↓

Output:        [Normal = 0.12,  Attack = 0.88]

↓

Decision:      88% probability of ATTACK → take action

| Step 6 | Decision Engine — Alert / Drop / Log |
| --- | --- |

Based on the AI model output, the decision engine takes appropriate action. There are three main options depending on the severity and policy configuration.

| Option | Method | How It Works |
| --- | --- | --- |
| Log | syslog / SIEM | Write anomaly event to system log or send to security monitoring system |
| Alert | MQTT / Alarm | Publish alert message via MQTT broker or trigger hardware alarm |
| Drop Traffic | NFQUEUE / iptables | Route through iptables NFQUEUE — user space gives verdict ACCEPT/DROP |
| Drop (fast) | eBPF/XDP | Program XDP hook to drop at driver level — lowest latency enforcement |
| Drop (custom) | Kernel module | Custom qdisc or netfilter module for advanced policy enforcement |

Dropping Traffic via NFQUEUE

# Route traffic through netfilter queue

iptables -A FORWARD -j NFQUEUE --queue-num 0

# User space application:

# 1. Receives packet via NFQUEUE

# 2. Runs DPI + AI inference

# 3. Issues verdict:

#    nfq_set_verdict(qh, id, NF_ACCEPT, 0, NULL);  // allow

#    nfq_set_verdict(qh, id, NF_DROP, 0, NULL);    // block

# 4. Kernel enforces the verdict

# Complete Flow on IQ9 — Full Pipeline

NIC (Ethernet PHY / stmmac driver)

↓

Kernel RX (IRQ → NAPI → netif_receive_skb)

↓

AF_PACKET socket (kernel → user space bridge)

↓

User-space DPI application

↓

nDPI (protocol detection: HTTP / Modbus / IEC 61850 / Unknown)

↓

Feature extraction

→ Protocol ID, packet length, flow duration,

→ packet count, inter-arrival time, SYN/ACK ratio

↓

AI model inference (QNN runtime)

→ CPU backend  (development)

→ GPU backend  (medium performance)

→ HTP backend  (Hexagon NPU — production, lowest power)

↓

Decision engine

→ [Normal = 0.12, Attack = 0.88]

↓

┌──────────────────────────────────────────┐

│  Action based on confidence threshold:   │

│  > 0.8  → DROP + Alert                   │

│  > 0.5  → Alert + Log + Rate limit       │

│  < 0.5  → Log only                       │

└──────────────────────────────────────────┘

↓

Kernel enforcement / Logging / MQTT alert

# Example Use Case — Industrial Anomaly Detection

A concrete example showing the complete pipeline applied to detecting an industrial network attack.

| Step | Action | Example Data |
| --- | --- | --- |
| 1 | Capture Ethernet frame | Raw bytes from stmmac driver via AF_PACKET |
| 2a | Extract packet size | 1514 bytes |
| 2b | Extract flow duration | 23 ms |
| 2c | Extract protocol | Modbus TCP (port 502) |
| 3 | Convert to tensor | [502, 1514, 23, 10, 2.0, 0.95] |
| 4 | Feed into ONNX/QNN model | QNN runtime on Hexagon NPU |
| 5a | Normal result | [Normal = 0.95, Attack = 0.05] → pass |
| 5b | Attack result | [Normal = 0.12, Attack = 0.88] → DROP + alert |

# Can We Use NPU Directly for DPI?

The Hexagon NPU cannot do protocol parsing directly, but the combination of nDPI + QNN on NPU is where IQ9 truly shines.

| Task | nDPI | QNN on Hexagon NPU |
| --- | --- | --- |
| Protocol parsing (L2-L7) | Yes — this is nDPI's strength | No — NPU is not a protocol parser |
| Feature extraction | Partial — provides protocol ID | No — done in user space app |
| Pattern matching | Yes — known protocol signatures | No — not designed for this |
| Anomaly detection | No — rule-based only | Yes — ML model inference |
| Unknown attack detection | Limited | Yes — AI generalizes to new patterns |
| Inference speed | N/A | High — NPU optimized for matrix math |

Where IQ9 Shines nDPI handles what it is designed for: fast, accurate protocol classification (L2-L7). QNN on Hexagon NPU handles what AI is designed for: pattern recognition and anomaly detection. Together they form a complete, hardware-accelerated intelligent networking pipeline. No other single platform offers this combination of DPI + NPU acceleration in an embedded form factor.

# Summary — Why This Architecture Works

| Component | Role | Why Needed |
| --- | --- | --- |
| AF_PACKET / libpcap | Packet capture bridge | Get raw packets to user space for inspection |
| nDPI | Protocol classification | Identify application protocol up to Layer 7 |
| Feature extraction | Data preparation | Convert raw packets to AI-ready tensors |
| QNN on HTP/NPU | AI inference | Detect anomalies that rule-based DPI misses |
| Decision engine | Action enforcement | Drop, alert, or log based on AI confidence |
| eBPF/XDP (optional) | Fast enforcement | Drop at driver level — bypass kernel stack |

Networking + AI Use Case  |  DPI + QNN on IQ9 QCS9075  |  Roopak  |  2026