Networking + AI Use Case

DPI + AI on Qualcomm IQ9 (QCS9075)

XDP_REDIRECT + nDPI + QNN Runtime — Industrial Anomaly Detection

Embedded Linux Networking  |  eBPF/XDP  |  AI Inference  |  Roopak  |  2026

# Overview

This document describes a complete Networking + AI pipeline running on the Qualcomm IQ9 (QCS9075) platform. The architecture uses eBPF/XDP_REDIRECT instead of AF_PACKET for faster, zero-copy packet capture, combined with nDPI protocol classification and QNN AI model inference on the Hexagon NPU for intelligent industrial anomaly detection.

## Why XDP_REDIRECT Instead of AF_PACKET?

| Feature | AF_PACKET (Old) | XDP_REDIRECT (New — Used Here) |
| --- | --- | --- |
| Capture point | After full kernel stack | Before kernel stack (driver level) |
| Memory copy | Yes — kernel copies to user space | Zero copy — shared UMEM |
| Latency | ~50-200 microseconds | ~5-20 microseconds |
| CPU overhead | High — full stack traversal | Low — bypasses stack |
| Drop capability | Via iptables/NFQUEUE | Directly via XDP_DROP + BPF map |
| Feedback loop | Via iptables rules (slow) | Via BPF map update (instant) |
| Best for | Simple monitoring | High performance DPI + AI on IQ9 |

## Updated Architecture — XDP_REDIRECT Pipeline

Kernel (XDP hook at driver level)

↓

XDP_REDIRECT → AF_XDP socket (zero copy via UMEM)

↓

nDPI Classification

↓

Feature Extraction

↓

AI Model Inference (QNN runtime on Hexagon NPU)

↓

Decision Engine → BPF map update → XDP_DROP (instant enforcement)

# Pipeline Stage Overview

| Stage | Component | What it Does | Where it Runs |
| --- | --- | --- | --- |
| 1 | Kernel + XDP Hook | eBPF program intercepts packets at NIC driver level | Linux Kernel / Driver |
| 2 | XDP_REDIRECT → AF_XDP | Zero-copy redirect to user space via shared UMEM | Kernel → User space |
| 3 | nDPI | Protocol classification — HTTP, TLS, Modbus, IEC 61850 | User space library |
| 4 | Feature Extraction | Converts packet data to numerical tensor | User space app |
| 5 | QNN Runtime | AI model inference on HTP/NPU for anomaly detection | Hexagon NPU / CPU / GPU |
| 6 | Decision + BPF map | Update blocklist map → XDP drops future attack packets instantly | User space + XDP |

| Step 1 | Kernel — XDP Hook at Driver Level |
| --- | --- |

When a packet arrives at the NIC, the eBPF/XDP program intercepts it at the earliest possible point — before any kernel stack processing. This is significantly faster than AF_PACKET which taps packets after full kernel processing.

Ethernet PHY → MAC → stmmac Driver

↓

XDP Hook (eBPF program runs HERE — earliest possible point)

↓

├── Known safe traffic  → XDP_PASS  → normal kernel stack

├── Blocked IP (BPF map) → XDP_DROP  → dropped instantly

└── Needs DPI inspection → XDP_REDIRECT → AF_XDP socket

| XDP Action | When Used | Result |
| --- | --- | --- |
| XDP_PASS | Known safe traffic (whitelisted) | Goes to normal kernel TCP/IP stack |
| XDP_DROP | IP in AI-generated blocklist | Dropped at driver level — zero overhead |
| XDP_REDIRECT | Unknown / needs inspection | Sent to AF_XDP socket for DPI + AI |

| Step 2 | XDP_REDIRECT — Zero Copy Bridge to User Space |
| --- | --- |

XDP_REDIRECT sends the packet directly to an AF_XDP socket via shared UMEM (User Memory). The DMA buffer is shared between the NIC and user space — no memory copy occurs at all.

Option A — Redirect ALL Traffic for Inspection

/* eBPF program — redirect all traffic to AF_XDP DPI socket */

struct {

__uint(type, BPF_MAP_TYPE_XSKMAP);

__uint(max_entries, 64);

__type(key, int);

__type(value, int);

} xsks_map SEC(".maps");

SEC("xdp")

int xdp_dpi_redirect(struct xdp_md *ctx)

{

/* Redirect ALL packets to AF_XDP socket for DPI */

return bpf_redirect_map(&xsks_map, 0, XDP_PASS);

}

Option B — Selective Redirect (Smart Filtering — Recommended)

SEC("xdp")

int xdp_selective_dpi(struct xdp_md *ctx)

{

struct iphdr *ip = /* parse headers */;

/* Check AI blocklist — drop confirmed attack IPs instantly */

__u8 *blocked = bpf_map_lookup_elem(&blocklist, &ip->saddr);

if (blocked && *blocked == 1)

return XDP_DROP;

/* Industrial protocols → high priority DPI queue */

if (eth->h_proto == htons(0x88BA) ||   /* SV */

eth->h_proto == htons(0x88B8)) {   /* GOOSE */

return bpf_redirect_map(&xsks_map, 0, XDP_PASS);

}

/* TCP/UDP → redirect for AI anomaly detection */

return bpf_redirect_map(&xsks_map, 1, XDP_PASS);

}

User Space DPI App Reads from AF_XDP — Zero Copy

/* Create AF_XDP socket connected to shared UMEM */

struct xsk_socket *xsk = xsk_socket__create("eth0", 0, umem, ...);

while (1) {

poll(&fds, 1, -1);   /* wait for packet arrival */

/* Read directly from UMEM — ZERO COPY */

uint64_t addr = xsk_ring_cons__rx_desc(&xsk->rx, idx)->addr;

uint8_t *pkt  = xsk_umem__get_data(umem->buffer, addr);

/* pkt points directly to DMA buffer — no copy at all */

/* Pass raw bytes directly to nDPI */

ndpi_process_packet(ndpi_struct, &flow, pkt, pkt_len, ts);

}

| Step 3 | nDPI Classification |
| --- | --- |

The raw packet bytes from the AF_XDP UMEM buffer are passed directly to nDPI. nDPI inspects all layers from Ethernet up to Layer 7 application payload and builds flow state across multiple packets.

nDPI processing pipeline:

Parse Ethernet header (L2)

↓

Parse IP header (L3)

↓

Parse TCP/UDP header (L4)

↓

Inspect payload (L7 — application layer)

↓

Build flow state (across multiple packets)

↓

Output: Flow = HTTPS / Modbus / IEC 61850 / Unknown

| Category | Protocols Detected |
| --- | --- |
| Web / IT | HTTP, HTTPS, TLS, DNS, FTP, SMTP, SSH |
| Industrial | Modbus, IEC 61850, GOOSE, Sampled Values, DNP3, EtherNet/IP |
| Streaming | RTP, RTSP, Video streaming protocols |
| Security | Known malware signatures, C2 communication patterns |
| Unknown | Unclassified flows → sent to AI for deeper analysis |

| Step 4 | Feature Extraction — Packets to AI Tensor |
| --- | --- |

Instead of passing raw bytes to the AI model, structured numerical features are extracted from each flow. This converts network traffic into a numeric tensor the AI model can process efficiently.

| Feature | Example Value | Meaning |
| --- | --- | --- |
| Protocol ID | 91 | HTTPS (nDPI protocol number) |
| Packet Length | 1514 | Size of packet in bytes |
| Flow Duration | 23 ms | How long the flow has been active |
| Packet Count | 10 | Number of packets in this flow |
| Avg Inter-Arrival Time | 2 ms | Time between consecutive packets |
| SYN/ACK Ratio | 0.95 | Ratio of SYN to ACK — attack indicator |
| Payload Entropy | 7.8 | Randomness of payload — high = encrypted/attack |

Network traffic → Numeric tensor

Example tensor: [91, 1514, 23, 10, 2.0, 0.95, 7.8]

│    │    │   │    │    │     │

│    │    │   │    │    │     └── payload entropy

│    │    │   │    │    └──────── SYN/ACK ratio

│    │    │   │    └───────────── inter-arrival time

│    │    │   └────────────────── packet count

│    │    └────────────────────── flow duration (ms)

│    └─────────────────────────── packet length

└──────────────────────────────── protocol ID

| Step 5 | AI Model — QNN Runtime on IQ9 |
| --- | --- |

The feature tensor is fed into an AI model running via the Qualcomm Neural Network (QNN) runtime. The Hexagon NPU (HTP) provides hardware-accelerated inference with the lowest power consumption.

| Backend | Hardware | Performance | Best For |
| --- | --- | --- | --- |
| CPU | ARM Cortex-A55 | Baseline | Development, debugging |
| GPU | Adreno GPU | Medium | Parallel inference tasks |
| HTP (Hexagon NPU) | Hexagon DSP | Highest — lowest power | Production deployment on IQ9 |

Example Model Output — Anomaly Detection

Input tensor:  [91, 1514, 23, 10, 2.0, 0.95, 7.8]

↓

QNN Model Inference (Hexagon NPU / HTP)

↓

Output:        [Normal = 0.12,  Attack = 0.88]

↓

Decision:      88% confidence ATTACK

→ Update BPF blocklist map

→ XDP drops future packets from this IP instantly

| Step 6 | Decision Engine + BPF Map Feedback Loop |
| --- | --- |

After the AI model gives a verdict, the decision engine takes action. The key improvement over AF_PACKET is the BPF map feedback loop — future packets from confirmed attack sources are dropped instantly at the XDP driver level without any user space involvement.

| AI Confidence | Action | Enforcement Method |
| --- | --- | --- |
| > 0.8 (High) | DROP + Alert + Log | Update BPF blocklist map → XDP_DROP at driver |
| > 0.5 (Medium) | Alert + Log + Rate limit | NFQUEUE or iptables rate limiting |
| < 0.5 (Low) | Log only | Write to syslog / SIEM for review |

BPF Map Feedback Loop — Instant Blocking

/* After AI says Attack (confidence > 0.8) */

/* Update BPF blocklist map from user space */

__u32 attacker_ip = flow->src_ip;

__u8  block       = 1;

bpf_map_update_elem(map_fd, &attacker_ip, &block, BPF_ANY);

/* Next packet from this IP hits XDP hook: */

/* XDP checks blocklist map → finds entry → XDP_DROP */

/* Dropped at driver level — never enters kernel stack! */

Other Action Options

| Option | Method | How It Works |
| --- | --- | --- |
| Log | syslog / SIEM | Write anomaly event to system log or security monitoring |
| Alert | MQTT / Alarm | Publish alert via MQTT broker or trigger hardware alarm |
| BPF blocklist | BPF map update | Instant XDP_DROP for future packets — no stack overhead |
| NFQUEUE | iptables hook | User space issues ACCEPT/DROP verdict — kernel enforces |
| Rate limit | iptables / tc | Limit bandwidth from suspicious source |

# Complete Flow on IQ9 — Full XDP Pipeline

NIC (stmmac driver / QCS9075)

↓

XDP Hook (eBPF — driver level, before kernel stack)

│

├── Blocked IP in BPF map? → XDP_DROP (instant, zero overhead)

│

├── Known safe → XDP_PASS → normal kernel TCP/IP stack

│

└── Needs inspection → XDP_REDIRECT → AF_XDP socket (UMEM)

↓

nDPI (protocol detection)

HTTP / Modbus / IEC 61850 / Unknown

↓

Feature extraction

[protocol, length, duration, count,

inter-arrival, SYN/ACK ratio, entropy]

↓

QNN runtime (AI inference)

→ CPU  (development)

→ GPU  (medium performance)

→ HTP  (Hexagon NPU — production)

↓

Decision Engine

[Normal=0.12, Attack=0.88]

↓

┌──────────────────┴──────────────────┐

│                                      │

Attack (>0.8)                         Normal (<0.5)

Update BPF blocklist map                      Log only

XDP_DROP future packets                       Continue monitoring

Alert via MQTT/syslog

# Example — Industrial Anomaly Detection

| Step | Action | Example Data |
| --- | --- | --- |
| 1 | XDP intercepts Ethernet frame | Raw bytes at NIC driver level — zero copy |
| 2a | Extract packet size | 1514 bytes |
| 2b | Extract flow duration | 23 ms |
| 2c | nDPI identifies protocol | Modbus TCP (port 502) |
| 3 | Convert to tensor | [502, 1514, 23, 10, 2.0, 0.95] |
| 4 | Feed into QNN model (NPU) | Hexagon HTP backend |
| 5a | Normal result | [Normal=0.95, Attack=0.05] → pass, log |
| 5b | Attack result | [Normal=0.12, Attack=0.88] → BPF blocklist → XDP_DROP |

# Can We Use NPU Directly for DPI?

The Hexagon NPU cannot perform protocol parsing directly, but combining nDPI protocol classification with QNN on NPU is exactly where IQ9 delivers its unique advantage.

| Task | nDPI | QNN on Hexagon NPU |
| --- | --- | --- |
| Protocol parsing (L2-L7) | Yes — nDPI's core strength | No — NPU is not a packet parser |
| Feature extraction | Partial — provides protocol ID | No — done in user space app |
| Known protocol matching | Yes — signature-based | No — not designed for this |
| Anomaly detection | Limited — rule-based only | Yes — ML generalizes to new attacks |
| Unknown attack detection | No | Yes — AI finds patterns in unknown traffic |
| Inference acceleration | N/A | Yes — NPU optimized for tensor operations |

Where IQ9 Shines — Unique Combination XDP_REDIRECT captures packets at driver level with zero copy — lowest possible latency. nDPI classifies protocols accurately up to Layer 7 — fast, rule-based classification. QNN on Hexagon NPU runs AI inference with hardware acceleration and minimal power. BPF map feedback loop enforces AI decisions at XDP level — instant, no stack overhead. No other embedded platform offers this complete XDP + DPI + NPU pipeline.

# Summary — Complete Architecture

| Component | Role | Advantage over AF_PACKET approach |
| --- | --- | --- |
| XDP Hook + eBPF | Intercepts packets at driver level | Before kernel stack — no skb overhead |
| XDP_REDIRECT + AF_XDP | Zero-copy packet delivery to user space | No memory copy — shared UMEM |
| nDPI | L7 protocol classification | Accurate flow-based detection |
| Feature Extraction | Raw packets → AI tensors | Structured input for AI model |
| QNN on HTP/NPU | AI anomaly detection | Hardware-accelerated inference on IQ9 |
| BPF map blocklist | Instant attack enforcement | XDP_DROP at driver — no stack cost |

DPI + AI on IQ9  |  XDP_REDIRECT + nDPI + QNN  |  Roopak  |  2026