# Networking (DPI + AI) Use case

Typical architecture:

Kernel (packet capture)

↓

User-space DPI

↓

Feature extraction

↓

AI Model inference (QNN runtime)

↓

Decision engine

You can:

- Capture packets using AF_PACKET or DPDK

- Extract L7 features

- Pass features to QNN model

- Get anomaly prediction

# Example Use Case (Industrial Anomaly Detection)

- Capture Ethernet frames

- Extract:

  - Packet size

  - Flow duration

  - Protocol

- Convert to tensor

- Feed into ONNX/QNN model

- If anomaly → drop packet / raise alert

# How It Would Fit Your Networking + AI Work

Since you previously discussed:

- Packet inspection

- DPI

- Anomaly detection

- IQ9 AI acceleration

Typical architecture on IQ9:

Kernel (packet capture)

↓

libpcap / AF_PACKET

↓

nDPI classification

↓

Feature extraction

↓

AI model (QNN runtime)

↓

Alert / Drop / Log

So yes — nDPI + AI model on IQ9 is absolutely possible.

# 🔹 1️⃣ Kernel (Packet Capture)

When a packet arrives:

Ethernet PHY → MAC → Driver → Linux Network Stack

Inside kernel:

- IRQ fires

- NAPI polls

- netif_receive_skb()

- Packet enters RX path

At this point, packet is inside kernel space.

Now you have two choices:

- Let it go normally to TCP/IP stack

- Or copy/redirect it to user space for inspection

# 🔹 2️⃣ libpcap / AF_PACKET (Getting Packet to User Space)

This is the bridge between kernel space → user space.

Two common methods:

### ✅ AF_PACKET (Raw Socket)

socket(AF_PACKET, SOCK_RAW, htons(ETH_P_ALL));

This lets your program receive raw Ethernet frames directly.

It taps packets before full TCP/IP processing.

### ✅ libpcap (Wrapper Library)

libpcap is built on top of AF_PACKET.

Example:

pcap_open_live("eth0", 65535, 1, 1000, errbuf);

So:

Kernel copies packet → User space buffer

Now your DPI application sees raw packet bytes.

# 🔹 3️⃣ nDPI Classification

Now packet is in user space.

You pass packet to:

nDPI

nDPI does:

- Parse Ethernet

- Parse IP

- Parse TCP/UDP

- Inspect payload (Layer 7)

It detects:

- HTTP

- TLS

- Modbus

- IEC 61850

- SSH

- etc.

It builds flow state, not just single packet inspection.

So after some packets, it says:

Flow = HTTPS

Flow = Modbus

Flow = Unknown

# 🔹 4️⃣ Feature Extraction

Now instead of raw packet bytes, you extract structured features.

Example:

| Feature | Meaning |
| --- | --- |
| Protocol ID | HTTPS = 91 |
| Packet length | 1514 |
| Flow duration | 23 ms |
| Packet count | 10 |
| Avg inter-arrival time | 2 ms |
| TLS SNI | google.com |

These become numerical inputs for AI model.

So you convert:

Network traffic → Numeric vector (tensor)

Example tensor:

[91, 1514, 23, 10, 2.0]

# 🔹 5️⃣ AI Model (QNN Runtime on IQ9)

Now this tensor goes into AI model running via:

Qualcomm AI Engine (QNN runtime)

Backends:

- CPU

- GPU

- HTP (Hexagon NPU)

The model could be:

- Anomaly detection

- Malware classifier

- Industrial attack detector

- Flow risk scorer

Example output:

[Normal = 0.12, Attack = 0.88]

So AI decides:

→ 88% probability attack

# 🔹 6️⃣ Alert / Drop / Log

Now your application takes action:

### Option A: Log

Write to syslog

Send to SIEM

### Option B: Alert

Send MQTT message

Trigger alarm

### Option C: Drop Traffic

If you want to drop:

You cannot drop directly from user space unless:

- Use NFQUEUE (iptables hook)

- Use eBPF/XDP

- Or use custom kernel module (like your qdisc work)

For example:

iptables -A FORWARD -j NFQUEUE --queue-num 0

User space inspects → verdict ACCEPT/DROP → kernel enforces

# 🔥 Complete Flow on IQ9

NIC

↓

Kernel RX

↓

AF_PACKET socket

↓

User-space DPI app

↓

nDPI (protocol detection)

↓

Feature extraction

↓

AI model (QNN on NPU)

↓

Decision

↓

Kernel enforcement / Logging

# Advanced: Can We Use NPU for DPI?

Not directly.

But:

- nDPI → protocol detection

- Extract features

- Feed features to AI model running on Hexagon NPU (QNN)

That’s where IQ9 shines.